How to read D01 file.

Please refer to TestAppDecDLL project

  - recfile2.h

  - d01.cpp

  - decoder.cpp

  - Frmdef.h

Open File:

LPRECFILE2 rcf_Open2(const char *pszFileName, int *errcode);

When the function returns NULL, errcode can be:

>= 0: unable to open the file and the value is from GetLastError

-1: not enough memory

Read one video frame

BOOL rcf_ReadVideoFrame2(LPRECFILE2 hFile, char **pBuf, int *pSize, __int64 *pAddr);

Close File:

void rcf_Close2(LPRECFILE2 hFile);

Header:

typedef struct tagADV_HEADER

{

BYTE    sync[4];

BYTE    header_size;

BYTE    header_version;

WORD    header_crc;

WORD    source;			// Camera Source

WORD    seq;

DWORD   size;

DWORD   time;

DWORD   flag;

DWORD   type;

WORD    data_crc;

DWORD   pointer[2];

DWORD   reserved[4];

} ADV_HEADER, * LPADV_HEADER;

type:

LOWORD (type) - specify frame type

#define ADVH_TYPE_NULL              	0x0000L     // for refresh link

#define ADVH_TYPE_H263              		0x0001L

#define ADVH_TYPE_MJPEG             	0x0002L

#define ADVH_TYPE_JPEG            	 	0x0003L

#define ADVH_TYPE_G723            	  	0x0004L

#define ADVH_TYPE_GPI               		0x0005L

#define ADVH_TYPE_G721            		0x0006L

#define ADVH_TYPE_POS               		0x0007L

#define ADVH_TYPE_XVD4              	0x0008L

#define ADVH_TYPE_VWEB		        	0x0009L

#define ADVH_TYPE_PENTA		        	0x000AL	    //VP-101

#define ADVH_TYPE_ADPCM             	0x000BL     //VP-101, audio frame

#define ADVH_TYPE_ADPCM1	        		0x000CL     //VP-504, SW ADPCM audio frame

#define ADVH_TYPE_SKL4              		0x000DL     //VP-504, FW1.4

LOBYTE(HIWORD(type)) - Subtype definition for H263/MJPEG/JPEG

#define ADVH_STYPE_VIDEO_NULL       	0x00L

#define ADVH_STYPE_VIDEO_640x480    	0x01L

#define ADVH_STYPE_VIDEO_320x240    	0x02L

#define ADVH_STYPE_VIDEO_640x240    	0x03L

#define ADVH_STYPE_VIDEO_720x480    	0x04L	//VP-101

#define ADVH_STYPE_VIDEO_360x240    	0x05L	//VP-101